WWF-Canada Carbon Training

Soil Carbon Sample Collection — In-Field Skill Checklist

Participant Name: ________________________________    Date: _______________    Site: _______________

Instructions: Work through each skill with your trainer. Check the left ☐ when you have practised the skill; check the right ✔ column when your trainer confirms competency. Both soil core and soil pit methods are included — complete the method(s) applicable to your site.

|  | Skill | Description / Key Actions | Complete ✔ |
| --- | --- | --- | --- |
| SECTION 1 — Site Selection & Sampling Design |  |  |  |
| Study Area & Plot Layout |  |  |  |
| ☐ | Define study area | Identify the geographic extent of the project area. Consider size, variation in vegetation and soil type across the landscape. | ☐ |
| ☐ | Survey soil depths | Use a soil auger or soil probe to measure depth-to-substratum at every 10–25 m grid intersection across the study site. | ☐ |
| ☐ | Establish plots | Set up plots within the site to capture variation in soil depth. Use digital grid overlay or compass + measuring tape method. | ☐ |
| ☐ | Survey within plots | Within each 10 m × 10 m plot, record soil depth at every 1 m intersection (100 measurements per plot minimum). | ☐ |
| ☐ | Select coring locations | Based on depth survey, identify coring sites that represent the range of soil depths across the plot. | ☐ |
| Nested Plot Design (if collecting multiple carbon pools) |  |  |  |
| ☐ | Plan nested plots | Map out large (20×20 m), medium (10×10 m), and small (1×1 m) plots before fieldwork begins. | ☐ |
| ☐ | Sequence sampling | Complete all vegetation surveys BEFORE any soil sampling — soil coring is a destructive technique. | ☐ |
| SECTION 2 — Site Preparation |  |  |  |
| Documentation & GPS |  |  |  |
| ☐ | Record CoreID | Assign a systematic unique identifier to each core (e.g., PE-01-B = Location–Site–Sample). Record in field notebook. | ☐ |
| ☐ | Record coordinates | Log latitude, longitude, and elevation of the coring site using a GPS waypoint. Record datum used. | ☐ |
| ☐ | Record date/time/weather | Note date, time, and weather conditions on the datasheet. Conditions can affect sample quality. | ☐ |
| Photo Series Protocol (14 photos per site) |  |  |  |
| ☐ | Photo – straight down | Photograph straight down from coring spot to document ground vegetation. | ☐ |
| ☐ | Photo – straight up | Photograph straight up to document canopy cover (or sky if open). | ☐ |
| ☐ | Photos – each cardinal direction | For each of N, S, E, W: take 3 photos — (a) parallel with ground, (b) 45° up, (c) 45° down. Total = 12 directional photos. | ☐ |
| SECTION 3 — Sampling |  |  |  |
| Method A — Soil Core (preferred where possible) |  |  |  |
| ☐ | Prepare coring area | Select a flat area near the coring spot. Lay down a tarp and organize all equipment before starting. | ☐ |
| ☐ | Insert the corer | Push corer straight into soil. Use serrated edge for roots; minimize compaction throughout. Depth guided by prior probe survey. | ☐ |
| ☐ | Twist & release | Once fully inserted, twist and jiggle the corer to shear the base of the sample from the surrounding sediment. | ☐ |
| ☐ | Extract the corer | Remove the corer while keeping upward pressure on the bottom to prevent the core from sliding out. | ☐ |
| ☐ | Sleeve the core | Turn corer horizontal; slide a longitudinally-cut plastic sleeve around it to capture the sample during extraction. | ☐ |
| ☐ | Push out & cap | Use the core extraction tool to push the sample into the sleeve. Cap both ends of the tube; secure with duct tape. | ☐ |
| ☐ | Measure & record | Measure the length of the extracted core AND the depth of the hole. Record both. Note: hole depth = true core depth (correct for compaction in lab). | ☐ |
| ☐ | Label & store | Label with CoreID, date, top/bottom orientation. Place upright in a cooler on ice for transport. | ☐ |
| Method B — Soil Pit |  |  |  |
| ☐ | Dig the pit | Excavate a hole ≥ 0.5 m wide, deep enough to expose all soil horizons down to the substratum or desired depth. | ☐ |
| ☐ | Describe horizons | For each visible soil layer, record: depth interval (top/bottom cm), colour (Munsell if possible), and texture (sand/silt/clay feel). | ☐ |
| ☐ | Classify horizons | Assign Canadian System of Soil Classification (CSSC) horizon designations: O (>17% OC), A (surface mineral), B (subsoil), C (substratum), W (water table), R (bedrock). | ☐ |
| ☐ | Collect bulk density samples | Use a soil sampling ring or bulk density disk (known volume) to collect a sample from the middle of each horizon. | ☐ |
| ☐ | Label & bag samples | Place each bulk density sample in a labelled resealable bag. Additional bulk samples from each layer can be bagged separately for %C analysis. | ☐ |
| ☐ | Store in cooler | Keep samples cold in a cooler for transport. Avoid sample disturbance. | ☐ |
| Method C — Shallow Soils (Canadian Shield / rocky sites only) |  |  |  |
| ☐ | Use frame/mould | Place the 0.25 m × 0.25 m frame on the ground and excavate all material within it. | ☐ |
| ☐ | Collect entire sample | Collect all excavated soil into a labelled resealable bag in one pass (no distinct layers). | ☐ |
| ☐ | Determine volume | Fill the excavated hole with water (or sand) of known volume to determine the sample volume used for bulk density. | ☐ |
| ☐ | Record name & volume | Note unique sample name and measured volume in field notebook. | ☐ |
| SECTION 4 — Lab Processing & Analysis |  |  |  |
| Lab Preparation |  |  |  |
| ☐ | Contact lab in advance | Coordinate with the receiving lab before fieldwork. Confirm their requirements for packaging, labelling, and delivery. | ☐ |
| ☐ | Core sectioning | In the lab, section cores by observed soil layer type (or at fixed depth intervals). Record top and bottom of each section in cm. | ☐ |
| Analyses (confirm with lab which are needed for your project) |  |  |  |
| ☐ | Moisture content | Weigh fresh sample (wet weight). Oven-dry at 105°C. Reweigh. Difference = moisture content. Required before LOI. | ☐ |
| ☐ | Dry bulk density | Measure volume of sample (from core diameter or ring volume). Dry bulk density (g/cm³) = dry mass / sample volume. | ☐ |
| ☐ | LOI₅₅₀ (organic matter) | Combust dried sample in muffle furnace at 550°C for ≥ 4 hours. Loss of mass = organic matter fraction. | ☐ |
| ☐ | Carbon conversion | Convert LOI to organic carbon: %C_org ≈ LOI₅₅₀ × 0.5 (organic matter is ~50% C by mass). | ☐ |
| ☐ | Alternative %C methods | If high inorganic fraction: consider IR spectroscopy for organic C, acid dissolution for inorganic C, or CHN elemental analyser for total C. | ☐ |
| SECTION 5 — Calculations & Scaling |  |  |  |
| Per-Subsection Calculations |  |  |  |
| ☐ | SOC density | Eq. 1: SOC density (g/cm³) = dry bulk density × (%C_org / 100). Repeat for each core subsection or soil layer. | ☐ |
| ☐ | Carbon stock per subsection | Eq. 2: Carbon stock (g/cm²) = SOC density × depth interval (cm). Depth interval = bottom depth – top depth. | ☐ |
| Per-Core / Per-Profile |  |  |  |
| ☐ | Sum subsections | Eq. 3: Carbon stock of core (g/cm²) = Σ carbon stocks of all subsections. Subsections must cover 100% of core depth. | ☐ |
| ☐ | Convert units | Eq. 4: Carbon stock (kg/m²) = core carbon stock (g/cm²) × 10. (1 g/cm² = 10 kg/m²) | ☐ |
| Scaling to Site & Study Area |  |  |  |
| ☐ | Average carbon stock – site | Eq. 5: Average site stock (kg/m²) = Σ core stocks / number of cores at site. | ☐ |
| ☐ | Total carbon – site | Eq. 6: Total site carbon (kg) = average site stock (kg/m²) × site area (m²). | ☐ |
| ☐ | Average carbon stock – study area | Eq. 7: Average study area stock (kg/m²) = Σ site total stocks (kg) / Σ site areas (m²). | ☐ |
| ☐ | Total carbon – study area | Eq. 8: Total study area carbon (kg) = average study area stock × study area size (m²). Multiply by 3.67 to convert kg C → kg CO₂e. | ☐ |