WWF-Canada Carbon Training

Peat Core Data Sheet

Field sheet for peatland soil carbon — bogs, fens and swamps. Field names match the Wetland Carbon Calculator tabs 1. Plot & Site Log, 2. Core Log and 3. Peat Data, so transcription is a straight copy.

Project name:  ______________________________________________     Page ______ of ______

| 1. PLOT & SITE LOG — one block per plot; fill this in FIRST |  |  |
| --- | --- | --- |
| Plot ID: | Site ID: | Study area: |
| Location: | Date: | Recorder: |
| Latitude: | Longitude: | GNSS accuracy (m): |
| Datum (e.g. WGS84): | Elevation (m): | Plot area (m²): |
| Peatland-specific — none of these are optional |  |  |
| Wetland type (bog / fen / swamp / marsh / other): ______________________________________ |  |  |
| Microform at the coring point (hummock / hollow / lawn): ______________________________________ |  |  |
| Vegetation zone (e.g. Sphagnum lawn, sedge lawn, shrub fen, treed swamp, lagg margin): ______________________________________ |  |  |
| Water table depth (cm) — measured DOWN from the peat surface; + = below surface, − = standing water above: ______________________________________ |  |  |
| Weather / conditions: ______________________________________ |  |  |

| 2. CORE LOG — one block per core |  |
| --- | --- |
| Recovery ratio = core length recovered ÷ bore depth. Below 0.90 the chamber probably did not fill — material is MISSING, which is not the same as compaction. Do not apply a compaction correction. |  |
| Core ID: | Surface compaction (cm): |
| Coring date: | Depth to mineral contact (cm): |
| Latitude: | Reached mineral contact? YES ☐ NO ☐ |
| Longitude: | von Post at base (H1–H10): |
| Drives taken (50 cm each): | Chamber length (cm): |
| Bore depth (cm): | Chamber diameter (cm): |
| Core length recovered (cm): | Photo series taken (14)? YES ☐ NO ☐ |
| Reason for refusal (buried wood / rock / gravel / rods bending / reached contact): |  |
| Core notes — colour and texture changes, gradual or distinct, wood, gaps, water saturation, drive boundaries: |  |

3.  PEAT DATA — core sections

Core ID:  ____________________        Sections cut at:  1 cm  ☐    2 cm  ☐    5 cm  ☐    at distinct layers  ☐

Bulk density basis the lab reports:   whole sample ☐      fine fraction (<2 mm) ☐      not yet confirmed ☐

Record TOP depth and BOTTOM depth, not thickness — the calculator derives thickness and checks for gaps and overlaps. Bulk density, water content, LOI and organic carbon % come back from the lab and are typed straight into the calculator; they are not field measurements. Note the DRIVE each section came from — sections straddling a drive boundary are the ones most likely to be wrong.

| Section ID | Drive # | Top depth (cm) | Bottom depth (cm) | von Post | Colour · texture · wood · gaps · saturation · notes |
| --- | --- | --- | --- | --- | --- |

Within-plot peat depth survey — 10 × 10 m plot, 1 m grid

Plot ID:  ____________________        Probe depths in cm.        121 points.

Choose the coring point at the MEDIAN of these depths, not the mean and not the deepest — one very deep reading is usually a hole between hummocks or a probe that punched into soft mineral. Keep this page: pooled across plots these depths are the variability prior for your next campaign.

| m ↓ / → | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |

Median depth:  ________ cm         Coring point chosen at:  x = ____ m,  y = ____ m,  depth ________ cm

If the coring point is NOT at the median depth, say why (tree / root mass / surface log / water too deep / ground unstable):

___________________________________________________________________________________________________

Probe depth is not core depth. A probe can stop on buried wood or a dense layer well above the true mineral contact, and can punch through soft mineral and overstate depth. Record both — Bore depth and Depth to mineral contact are separate fields on page 1.

von Post humification scale — field reference

Squeeze a handful of WET peat and watch what comes out between your fingers.

| Class | Degree of decomposition | What you see when you squeeze a handful of WET peat |
| --- | --- | --- |
| H1 | Completely undecomposed | Clear, colourless water. Plant remains easily identifiable. |
| H2 | Almost undecomposed | Clear, yellowish water. Plant remains still identifiable. |
| H3 | Very weakly decomposed | Slightly turbid brown water. NO peat escapes between the fingers. |
| H4 | Weakly decomposed | Strongly turbid brown water. No peat escapes. Residue mushy. |
| H5 | Moderately decomposed | Very turbid water, a LITTLE peat escaping. Structure evident but indistinct. |
| H6 | Fairly well decomposed | About ONE THIRD escapes between the fingers. Residue very mushy. |
| H7 | Strongly decomposed | About HALF escapes. Plant structure barely discernible. |
| H8 | Very strongly decomposed | About TWO THIRDS escapes. Only resistant remains — roots, fibres. |
| H9 | Almost completely decomposed | Nearly all escapes as a fairly uniform paste. |
| H10 | Completely decomposed | ALL escapes between the fingers. No free water visible. |

H1–H3 = fibric peat (Oi)      ·      H4–H6 = hemic, “mucky peat” (Oe)      ·      H7–H10 = sapric, “muck” (Oa)

The more decomposed the material, the LESS carbon per unit weight — but also the HIGHER its bulk density, and those two partly cancel. Do not assume a muck layer holds less carbon per centimetre than a fibric one. Measure it.

Three transitions worth a note every time

| Frame | Where | The note that matters |
| --- | --- | --- |
| A | 0–50 cm, Sphagnum bog | The depth where green, LIVING Sphagnum turns to brown peat. This is the vegetation/peat boundary. |
| B | mid-core, 50–100 cm | Dark mucky peat at the top grading to lighter peat below, and the depth of any significant WOOD. |
| C | at the base | The depth of the peat → mineral transition, the texture and colour of the mineral layer, and the depth where coarse fragments become common. |

Sources: Measuring Carbon in Peat Soils: A Supplemental Guide (WWF-Canada, 2024). Core-length, surface-compaction, bore-depth and cylinder-diameter fields follow Bansal et al. (2023), Practical Guide to Measuring Wetland Carbon Pools and Fluxes, Wetlands 43:105, p.22.